ANTARCTIC VESSEL NAVIGATION

AI/ML Decision-Support Platform for Safe & Fuel-Efficient Sea Routes

Project Reference Document

Special Focus: MODEL 3 — Safe and Fuel-Efficient Route Planning

|  | MODEL 1 Future Sea-Ice Prediction → MODEL 2 Future Iceberg Prediction → MODEL 3 Route Planning (Focus of this Document) |  |
| --- | --- | --- |

Prepared for internal group understanding and presentation preparation.

# Table of Contents

# 1. Our Project

Our project is an AI/ML-enabled decision-support platform for forecasting Antarctic sea-ice concentration, predicting iceberg trajectories, and identifying safe and fuel-efficient navigation routes for research vessels.

Antarctic waters are difficult to navigate because they contain a combination of hazards, including:

- Sea ice

- Icebergs

- Strong winds

- Large waves

- Ocean currents

- Changing environmental conditions over time

Because of these hazards, a research vessel cannot simply travel in a straight line from its starting point to its destination. The system must first understand and predict future environmental conditions and iceberg locations, and then plan a safer route around the dangerous regions.

The project is organised into three major stages:

| Stage | Name | Role |
| --- | --- | --- |
| Model 1 | Future Sea-Ice Concentration Prediction | Predicts how much ice will cover an area in the future |
| Model 2 | Future Iceberg Trajectory Prediction | Predicts where icebergs are expected to be in the future |
| Model 3 | Safe and Fuel-Efficient Route Planning | Uses Model 1 and Model 2 outputs to plan the vessel's route |

# 2. Complete System in Simple Words

The whole project can be understood through three simple questions:

| Model | Question it answers |
| --- | --- |
| Model 1 | “What will the future sea-ice concentration look like?” |
| Model 2 | “Where is the iceberg expected to be in the future?” |
| Model 3 | “Considering predicted sea ice, predicted iceberg locations, weather/ocean conditions, and vessel information — which route should the ship take to reach its destination safely and efficiently?” |

## System Data-Flow Diagram

|  | HISTORICAL DATA |  |
| --- | --- | --- |

↓

| MODEL 1 Future SIC Prediction | MODEL 2 Future Iceberg Prediction |
| --- | --- |

↓

| Future SIC Data | Future Iceberg Positions |
| --- | --- |

↓

| Wind | Waves | Ocean Currents |
| --- | --- | --- |

↓  plus Vessel Information (start, destination, speed, fuel)

|  | MODEL 3 — ROUTE PLANNING Navigation Grid → Risk / Cost Calculation → Dangerous Cells Blocked A* Path Search evaluates the remaining safe cells |  |
| --- | --- | --- |

↓

|  | SAFE + FUEL-EFFICIENT RECOMMENDED ROUTE |  |
| --- | --- | --- |

Figure 2.1 — End-to-end data flow from historical data through Model 1 / Model 2 into Model 3.

# 3. Model 1 — Future Sea-Ice Concentration Prediction (Brief)

Model 1 predicts the future Sea Ice Concentration (SIC) for a geographical location. SIC represents how much of an area is covered by sea ice, usually expressed as a percentage.

| Location | Predicted SIC | Interpretation |
| --- | --- | --- |
| Location A | 20% | Mostly open water |
| Location B | 50% | Moderate ice cover |
| Location C | 90% | Heavily ice covered |

Higher SIC generally means more ice, which can make navigation more difficult. Model 1 produces future SIC values that are later used by Model 3 for route-risk assessment.

Scope note: This document does not go into the internal neural-network architecture of Model 1. The key point for the team is: Model 1 = Future Sea-Ice Condition Prediction, and its output feeds into Model 3.

# 4. Model 2 — Future Iceberg Trajectory Prediction (Brief)

Model 2 predicts the future position of an iceberg. Its important outputs are the predicted latitude, predicted longitude, and ideally the prediction time/date.

| Iceberg | Predicted Latitude | Predicted Longitude |
| --- | --- | --- |
| Iceberg 1 | -66.2 | 22.3 |
| Iceberg 2 | -67.1 | 24.8 |

In simple terms, Model 2 tells Model 3: “These are the locations where icebergs are expected to be in the future.”

Scope note: This document does not go into the internal architecture of Model 2 in detail. The key point for the team is: Model 2 = Future Iceberg Location Prediction, and its output feeds into Model 3.

# 5. Why Model 3 Is Required

Knowing where an iceberg will be is not enough on its own. Model 2 can tell us that an iceberg will be at a certain location, but the vessel still needs an answer to a different question: “Where should I go instead?” This is exactly the job of Model 3.

Model 3 takes the predicted hazards from Model 1 and Model 2 and searches for an alternative route that avoids them.

## Illustrative Example

|  |  | DEST |  |  |
| --- | --- | --- | --- | --- |
| OK | OK | OK | OK | OK |
| OK | ICE | ICE | OK | OK |
| OK | ICE | ICE | OK | OK |
|  |  | START |  |  |

■ Safe cell      ■ Iceberg / blocked cell

Figure 5.1 — The direct path from START to DEST crosses predicted iceberg cells, so Model 3 must find a path around the danger area.

# 6. Model 3 — Main Purpose

Model 3 is the route-planning and optimisation stage of the project. It is not primarily another deep-learning prediction model — it is a decision-making process that combines predictions and rules to choose a route.

Its job is to:

- Receive future SIC from Model 1

- Receive predicted iceberg positions from Model 2

- Receive environmental / navigation information (wind, waves, currents)

- Receive vessel information (start, destination, speed, fuel)

- Build a navigation environment / grid

- Evaluate risk for each geographical cell

- Block dangerous cells

- Search possible paths across the grid

- Select a safe and relatively fuel-efficient path

- Produce the recommended route

Key statement: Model 1 predicts the ice condition, Model 2 predicts the iceberg location, and Model 3 decides how the vessel should navigate.

# 7. Data Required by Model 3

Model 3 combines environmental data, Model 1 / Model 2 outputs, and vessel information to build its navigation grid.

## 7.1 Environmental Inputs

| Field | Meaning | Source | Purpose in Model 3 |
| --- | --- | --- | --- |
| lat | Latitude of the location | Environmental dataset | Positions the cell on the navigation grid |
| lon | Longitude of the location | Environmental dataset | Positions the cell on the navigation grid |
| ice_conc | Sea Ice Concentration | Model 1 output | Used in sea-ice risk calculation |
| uo | Ocean current component (direction 1) | Environmental dataset | Used in current-risk calculation |
| vo | Ocean current component (direction 2) | Environmental dataset | Used in current-risk calculation |
| u10 | Wind component at ~10 m (direction 1) | Environmental dataset | Used in wind-risk calculation |
| v10 | Wind component at ~10 m (direction 2) | Environmental dataset | Used in wind-risk calculation |
| wave_height | Height of ocean waves | Environmental dataset | Used in wave-risk calculation |

## 7.2 Model Outputs Used as Inputs

- Future SIC from Model 1

- Predicted iceberg latitude and longitude from Model 2

## 7.3 Vessel Information

- Start latitude

- Start longitude

- Destination latitude

- Destination longitude

- Vessel speed

- Fuel consumption estimate

# 8. Our Historical Vessel Route Dataset

| Property | Value |
| --- | --- |
| Approximate records | 654,808 vessel-position records |
| Number of routes | 43 routes |

## 8.1 Columns

| Column | Meaning |
| --- | --- |
| latitude | Latitude of the recorded vessel position |
| longitude | Longitude of the recorded vessel position |
| timestamp | Date/time of the recorded position |
| speed | Vessel speed at that point in time |
| heading | Direction the vessel was facing |
| north_speed | Speed component in the north direction |
| east_speed | Speed component in the east direction |
| route_id | Identifier of the route the record belongs to |
| source_file | Original file the record was extracted from |

## 8.2 How This Dataset Helps Model 3

- Understanding historical vessel trajectories and where vessels have travelled

- Historical speed information

- Historical heading / direction information

- Travel-distance analysis

- Travel-time analysis using timestamps

- Understanding realistic vessel movement patterns

- Route validation — checking a planned route against what is realistic

- Comparing Model 3's recommended route with historical routes

Important limitation: This dataset does NOT directly predict future iceberg locations — it represents historical vessel movement only. It can help validate and understand navigation behaviour, but it should not replace future SIC, predicted iceberg locations, waves, wind, or ocean currents. In addition, the columns heading, north_speed, and east_speed have substantial missing values in parts of the dataset, so they should not be treated as mandatory inputs for every historical record.

## 8.3 Role in the Pipeline

|  | Historical Vessel Routes → Historical Navigation Patterns → Speed / Direction / Distance Analysis → Model 3 Validation & Navigation Analysis |  |
| --- | --- | --- |

# 9. Navigation Grid

Model 3 represents the navigable geographical area as a grid of cells (points). The ship does not make one giant decision from start to destination in a single step — it moves from one cell to a neighbouring cell, and Model 3 evaluates the possible neighbouring cells at every step.

## Example Grid

| OK | OK | OK | OK | OK |
| --- | --- | --- | --- | --- |
| OK | OK | ICE | ICE | OK |
| OK | OK | ICE | ICE | OK |
| OK | OK | OK | OK | OK |

Figure 9.1 — A simplified navigation grid. Longitude increases left to right; latitude increases bottom to top.

Each cell in the grid can carry the following attributes:

| Attribute | Description |
| --- | --- |
| latitude, longitude | Position of the cell |
| SIC | Sea-ice concentration at the cell (from Model 1) |
| wave height | Expected wave height at the cell |
| wind (u10, v10) | Wind conditions at the cell |
| ocean current (uo, vo) | Current conditions at the cell |
| distance from predicted iceberg | Nearest iceberg distance (from Model 2) |
| risk / cost | Combined score used by the path search |

# 10. How Model 3 Decides If a Cell Is Safe

For every navigation cell, Model 3 evaluates a combination of risk and cost:

- Sea Ice Risk

- Iceberg Risk

- Wave Risk

- Wind Risk

- Ocean Current Risk

- Fuel / Travel Cost

## Example Cells

| Cell | SIC | Iceberg Distance | Wave Height | Assessment |
| --- | --- | --- | --- | --- |
| Cell A | 20% | 80 km | 1.5 m | Relatively low risk |
| Cell B | 85% | 25 km | 4 m | Higher risk |
| Cell C | 95% | 10 km | 6 m | Dangerous |

Model 3 uses both hard safety rules (cells that are simply blocked) and soft cost/risk values (used to compare cells that are otherwise allowed).

# 11. Safety Thresholds

The current prototype uses the following thresholds:

| Condition | Threshold | Result if exceeded |
| --- | --- | --- |
| Sea Ice Concentration | Maximum safe SIC = 90% | Cell marked unsafe / blocked |
| Iceberg Distance | Minimum safe distance = 20 km | Cell marked unsafe / blocked |
| Wave Height | Maximum safe wave height = 5 m | Cell marked unsafe / blocked |

## Decision Logic

|  | CELL |  |
| --- | --- | --- |

↓

|  | Check conditions (SIC, iceberg distance, wave height) |  |
| --- | --- | --- |

| UNSAFE → BLOCK | SAFE → Assign cost, A* can evaluate |
| --- | --- |

Figure 11.1 — Every cell is first checked against hard safety rules before being scored.

Prototype assumption: These thresholds are current prototype/project values and should be validated against appropriate maritime and polar navigation guidelines before any real-world deployment.

# 12. Iceberg Avoidance — Deep Explanation

Model 2 predicts a set of iceberg positions, for example:

| Iceberg | Latitude | Longitude |
| --- | --- | --- |
| Iceberg 1 | -66.2 | 22.3 |
| Iceberg 2 | -67.1 | 24.8 |
| Iceberg 3 | -68.0 | 26.5 |

For every navigation cell, Model 3 calculates the geographical distance between that cell and the nearest predicted iceberg. This uses the Haversine distance formula, which calculates the shortest distance between two points on the surface of a sphere (the Earth), using their latitude and longitude. The result is a value called iceberg_distance_km.

## Example

| Cell | Iceberg Distance | Assessment |
| --- | --- | --- |
| Cell A | 10 km | Dangerous |
| Cell B | 25 km | Near safety boundary |
| Cell C | 60 km | Lower risk |
| Cell D | 100 km | Very low / near-zero iceberg risk under the prototype risk function |

Key idea: The route planner does not simply ask “Is there an iceberg?” It asks “How close is the predicted iceberg to this possible route cell?” — distance, not just presence, drives the risk score.

# 13. How Model 3 Decides the Direction

At every point, the vessel can potentially move to a neighbouring cell. The current route-planning implementation considers 8 neighbouring directions (also known as 8-connectivity):

| ↖ NW | ↑ N | ↗ NE |
| --- | --- | --- |
| ← W | SHIP | → E |
| ↙ SW | ↓ S | ↘ SE |

Figure 13.1 — The 8 candidate directions Model 3 evaluates from the vessel's current cell.

Model 3 does not have a fixed rule such as “always go north.” Instead, it evaluates every possible neighbouring cell's risk and cost, and compares them.

## Example Decision

| Direction | Iceberg Distance | SIC | Wave Height | Result |
| --- | --- | --- | --- | --- |
| Up | 10 km | 80% | 3 m | Very high risk → blocked |
| Right | 50 km | 30% | 2 m | Low risk |
| Left | 70 km | 20% | 2 m | Low risk |
| Down | — | — | 6 m | Blocked (wave too high) |

Model 3 will not select Up or Down because both are blocked. It compares Left and Right. If Right has a lower total cost and still leads effectively toward the destination, it is selected.

Important: The system does not predict a “direction” as a separate machine-learning output. The direction the vessel takes is simply the result of the path-search process comparing costs of neighbouring cells.

# 14. Cost Function

The current prototype combines the individual risk types into a single total cost, using the following weights:

| Risk Component | Weight |
| --- | --- |
| Sea Ice | 40% |
| Iceberg | 30% |
| Wave | 15% |
| Wind | 5% |
| Current | 5% |
| Fuel | 5% |

|  | Total Cost = 0.40 × Ice Risk + 0.30 × Iceberg Risk + 0.15 × Wave Risk + 0.05 × Wind Risk + 0.05 × Current Risk + 0.05 × Fuel Cost |  |
| --- | --- | --- |

In simple terms: higher risk leads to higher cost, and lower risk leads to lower cost. A* tries to find a path that reaches the destination while keeping the total accumulated cost as low as possible.

Prototype assumption: These are prototype weights. They can later be tuned and validated using experiments and domain knowledge, rather than being treated as final values.

# 15. A* Algorithm — Deep Explanation

A* (pronounced “A-star”) is a path-search and optimisation algorithm. It is not a deep-learning model. It searches for a path from the start point to the destination while considering:

- The cost already travelled so far

- The environmental risk/cost of each step

- The estimated remaining distance to the destination

Simplified idea: A* = “Find a good path to the destination while considering the cost of the path so far and the distance still remaining.”

## Path Comparison

| Path A → crosses iceberg BLOCKED | Path B → clear cells FEASIBLE, LOW COST | Path C → crosses high waves BLOCKED |
| --- | --- | --- |

Figure 15.1 — A* evaluates several candidate paths from START and selects a feasible, low-cost path such as Path B.

# 16. Hard Safety vs. Cost Optimisation

## Hard Safety

Some cells are considered unacceptable and are blocked outright, for example when:

- Iceberg distance < 20 km

- SIC > 90%

- Wave height > 5 m

## Soft Cost

Among the remaining safe cells, some are better than others. For example:

| Route | SIC | Wave Height | Iceberg Distance | Note |
| --- | --- | --- | --- | --- |
| Route A | 30% | 2 m | 80 km | Lower risk/cost |
| Route B | 70% | 4 m | 30 km | Higher risk/cost, but still technically allowed |

Both routes may be technically allowed under the hard safety rules, but Route A has a lower risk/cost, so A* prefers it over Route B.

# 17. Fuel-Efficient Routing

The system does not simply find the shortest geographical route. A shortest route may pass directly through high ice concentration, large waves, or icebergs, which is unsafe.

| Shortest Route (through hazards) UNSAFE | Alternative Route (around hazards) LONGER BUT SAFER |
| --- | --- |

Figure 17.1 — A longer but safer route can be preferable to the shortest but hazardous route.

Model 3 attempts to balance:

- Safety

- Environmental risk

- Travel distance

- Fuel consumption

Therefore, the best route is not necessarily the absolute shortest route — it is the route with the best overall balance of safety and efficiency.

# 18. Complete Model 3 Pipeline

|  | START + DESTINATION + Future SIC (Model 1) + Future Iceberg Positions (Model 2) + Wind + Wave + Current + Vessel Information |  |
| --- | --- | --- |

↓

|  | Create Navigation Grid For every cell: calculate iceberg distance, environmental risks, total cost |  |
| --- | --- | --- |

↓

|  | Apply safety rules → Block dangerous cells |  |
| --- | --- | --- |

↓

|  | Run A*: evaluate neighbouring cells, compare possible paths, select feasible low-cost path |  |
| --- | --- | --- |

↓

|  | Calculate route distance + Estimate fuel → Generate recommended route recommended_route.csv |  |
| --- | --- | --- |

Figure 18.1 — Complete Model 3 processing pipeline, from inputs to the final route file.

# 19. Final Output of Model 3

Model 3 produces a route file, for example recommended_route.csv, with columns such as:

| Column | Description |
| --- | --- |
| lat, lon | Position of each route point |
| ice_conc | Sea-ice concentration at that point |
| iceberg_distance_km | Distance to the nearest predicted iceberg |
| uo, vo | Ocean current at that point |
| u10, v10 | Wind at that point |
| wave_height | Wave height at that point |
| cell_cost | Calculated cost of that cell |

In addition, the following final route metrics can be reported:

- Number of route points

- Total route distance

- Estimated fuel consumption

- Average SIC along the route

- Minimum iceberg distance along the route

- Maximum wave height along the route

The final route can also be visualised on a map, showing the path from the start position to the destination around the blocked hazard cells.

# 20. Complete End-to-End Example (Illustrative)

Note: All values below are illustrative examples only, used to explain the concept. They are not real experimental results.

| Point | Latitude | Longitude |
| --- | --- | --- |
| Start | -65 | 20 |
| Destination | -70 | 30 |

In this example, Model 1 predicts high SIC in one region, and Model 2 predicts several icebergs in another region. Model 3 builds the navigation grid, blocks the dangerous cells, and A* evaluates the possible directions.

| DIRECT ROUTE START → through ICE → DEST BLOCKED / UNSAFE | RECOMMENDED ROUTE START → around hazards → DEST SAFE & FEASIBLE |
| --- | --- |

Figure 20.1 — The direct route is blocked by predicted hazards, so Model 3 recommends a route around them.

# 21. Role of All Three Models

| Model | Purpose | Key Output |
| --- | --- | --- |
| Model 1 | Future sea-ice concentration prediction | Future SIC |
| Model 2 | Future iceberg trajectory prediction | Future iceberg latitude / longitude |
| Model 3 | Safe and fuel-efficient route planning | Recommended route |

Model 3's inputs summarised: future SIC, predicted iceberg positions, wind, waves, ocean currents, vessel information, and historical route information where useful.

| MODEL 1 “What will the ice conditions be?” | MODEL 2 “Where will the iceberg be?” | MODEL 3 “Where should the ship go?” |
| --- | --- | --- |

# 22. Important Limitations

- Iceberg predictions contain uncertainty.

- Future SIC predictions contain uncertainty.

- Environmental datasets may have spatial/temporal mismatches.

- Prototype safety thresholds require validation.

- Fuel consumption is an estimate.

- Historical routes do not guarantee current safety.

- Real-world vessel navigation requires official maritime and polar navigation procedures.

- The system is a decision-support / research prototype, not an autonomous navigation system.

# 23. Final One-Page Summary

| MODEL 1 Future Sea-Ice Prediction “Where is ice?” | MODEL 2 Future Iceberg Prediction “Where is the iceberg?” | MODEL 3 Route Planning “Where should the ship go?” |
| --- | --- | --- |

↓

|  | Risk + Cost Calculation → Dangerous Cells Blocked → A* Path Search |  |
| --- | --- | --- |

↓

|  | SAFE + FUEL-EFFICIENT ROUTE |  |
| --- | --- | --- |

“Our system first predicts future sea-ice conditions and iceberg positions, then uses those predictions together with environmental conditions and vessel information to find a safer and more fuel-efficient route to the destination.”